Practical No. 6

Aim: Create a basic MANET implementation simulation for Packet animation and Packet Trace.

Steps:

- Open Omnet++ idle.

- Expand the inet folder > examples > manetrouting. Right click on manetrouting > New > Folder as MobileNet.

- Right Click on MobileNet > New > Network Description File (NED).

- Name the file as Net1 and Click Next.

- Select New Adhoc Mobility Wireless Network Wizard and Click next.

- Configure as follow.

- Click on Finish.

- To configure, Right click on inet folder > Run as > Run Configurations. Select Net1 > Browse the directory where the project is > Apply > Run.

Output:

Below is the code that will be available in source part of net1.ned once configured.

package inet.examples.manetrouting.MobileNet;

// numOfHosts: 10

// parametric: true

// static:     false

import inet.networklayer.autorouting.ipv4.IPv4NetworkConfigurator; import inet.nodes.inet.AdhocHost; import inet.world.radio.ChannelControl;

network Net1

{     parameters:

int numHosts;     submodules:

host[numHosts]: AdhocHost

{             parameters:

@display("r=,,#707070");

}

channelControl: ChannelControl

{             parameters:                 @display("p=60,50");

}

configurator: IPv4NetworkConfigurator

{

@display("p=140,50");

}

}

A file omnetpp.ini will be created with the following source code.

[General] network = Net1 #record-eventlog = true

#eventlog-message-detail-pattern = *:(not declaredOn(cMessage) and not declaredOn(cNamedObject) and not declaredOn(cObject))

*.numHosts = 10

num-rngs = 3 **.mobility.rng-0 = 1

**.wlan[*].mac.rng-0 = 2

#debug-on-errors = true

tkenv-plugin-path = ../../../etc/plugins

**.channelNumber = 0

# channel physical parameters

*.channelControl.carrierFrequency = 2.4GHz

*.channelControl.pMax = 2.0mW

*.channelControl.sat = -110dBm

*.channelControl.alpha = 2

*.channelControl.numChannels = 1

# mobility

**.host[*].mobilityType = "MassMobility"

**.mobility.constraintAreaMinZ = 0m

**.mobility.constraintAreaMaxZ = 0m

**.mobility.constraintAreaMinX = 0m

**.mobility.constraintAreaMinY = 0m

**.mobility.constraintAreaMaxX = 600m

**.mobility.constraintAreaMaxY = 400m

**.mobility.changeInterval = truncnormal(2s, 0.5s)

**.mobility.changeAngleBy = normal(0deg, 30deg)

**.mobility.speed = truncnormal(20mps, 8mps)

**.mobility.updateInterval = 100ms

# ping app (host[0] pinged by others)

*.host[0].pingApp[0].destAddr = ""

*.host[*].numPingApps = 1

*.host[*].pingApp[0].destAddr = "host[0]"

*.host[*].pingApp[0].startTime = uniform(1s,5s)

*.host[*].pingApp[0].printPing = true

# nic settings

**.wlan[*].bitrate = 2Mbps

**.wlan[*].mgmt.frameCapacity = 10

**.wlan[*].mac.address = "auto"

**.wlan[*].mac.maxQueueSize = 14

**.wlan[*].mac.rtsThresholdBytes = 3000B

**.wlan[*].mac.retryLimit = 7

**.wlan[*].mac.cwMinData = 7

**.wlan[*].mac.cwMinMulticast = 31

**.wlan[*].radio.transmitterPower = 2mW

**.wlan[*].radio.thermalNoise = -110dBm

**.wlan[*].radio.sensitivity = -85dBm

**.wlan[*].radio.pathLossAlpha = 2

**.wlan[*].radio.snirThreshold = 4dB